1

Output and Input Signal Sheet (ELSA001)
Project Engineering Department

88 Bangbon Vill  Kanjanapisek Rd. Bangbon, Bangbon, Bangkok Thailand 10150

	Dear
	Electrical and Control Supervisory Division, Project Engineering Department


	I would like to inform you that we have got the new project from 
	EMERSON

	and would like to cooperate in the electrical cabinet design and wiring for this project as well as itemize the detailed descriptions as follows:


Power Machine
	Seq.
	Equipment
	Specification
	Qty.
	Input/EA

(All)
	Status
	Output/EA
(All)
	Status

	1
	Dresser Roots Blower
	7.5kW,380V,3( 
Direct on-line start
	2
	4 (8)
	start

stop

overload

 active
	1 (2)
	Relay

	2
	6” Rotary Valve
	0.37kW,380V,3( 

Direct on-line start
	12
	4 (48)
	start

stop

overload

 active
	1 (12)
	Relay

	3
	10” Rotary Valve
	
	1
	4 (4)
	start

stop

overload

 active
	1 (1)
	Relay

	4
	Vibrator
	0.27kW,380V,3( 

Direct on-line start
	2
	0 (0)
	-
	1 (2)
	Relay


Input device
	Seq.
	Equipment
	Description
	Qty.
	Input/EA (All)
	Status

	1
	Vacuum switch
	Digital Input
	1
	1 (1)
	Logic switch

	2
	Capacitive Sensor
	Digital Input
	2
	1 (2)
	Logic switch

	3
	System Start
	Digital Input
	1
	6 (6)
	Logic switch


Output device

	Seq.
	Equipment
	Specification
	Qty.
	Output/EA (All)
	Status

	1
	Jet Pulse
	Dust Collector
	2
	1 (2)
	24 VDC Relay

	2
	Mimic
	Pilot Lamp
	19
	1 (19)
	24 VDC Relay

	3
	Buzzer
	Alarm
	1
	1 (1)
	24 VDC Relay


Miscellaneous device
	Seq.
	Equipment
	Specification
	Qty.
	Input/EA

(All)
	Status
	Output/EA

(All)
	Status

	1
	6” Butterfly Valve


	Solenoid Valve: 

3/2 type 24 VDC
	2
	2 (4)
	Limit: Left

Limit: Right
	1 (2)
	Coil


Front Panel
	Seq.
	Equipment
	Specification
	Qty.

	1
	Voltmeter
	0 – 500 VAC
	1

	2
	Ammeter
	Burden: 100A/5A 
	1

	3
	Emergency Stop
	Emergency Switch
	1

	4
	Selector Switch
	3( Selector Switch – Phase selection for voltage monitoring
	1


Programmable Logic Controller (PLC) Module
	Seq.
	Equipment
	Specification
	Qty.

	1
	Allen Bradley
	Master Module 

12 inputs / 12 outputs
	1

	2
	Allen Bradley
	Expansion Module
16 inputs / 16 outputs
	6


	For more information, please contact to:
	1. MR. XXXXXXX (his position – Leader/Manager)

	
	2. MR. XXXXXXX (his position – Sales Representative/Sales Project)


	Best Regards,
	Receive Date:
	

	
	Receive Time:
	

	Sales Representative/Sales Project’s Name

Sales Representative/Sales Project’s POSITION
Project Engineering Department
	Received By
	


Enclosure: 
Single Line Diagram   
1 Sheet


Sequential Condition
1 Sheet

